# Wireshark: UDP

## BSDSF22A001 – Muhammad Rehan

- Packet Number: 197

- Protocol Message:

5ae00c4b707b317da46b80a6d327074d98c3196abe2b93f1b78396d7f0b675c6369995a642b70832e85c2d62ed95c58c60fac1688de92113864ea414d3ecfa8ce2d3bbd9160bd3d3a71ebbc4b9bb0b518d948febfaeb8072ba7e1de8caaa8b47297143e296b153f8412f72775225f2a3f8ac247f3660b88273e33ef1c266280cbb027861d21ee2fd45f68b96f36cc98ba39e7ed42edbb27205becf91ea7ffa1dc5e4fd1265ee5a94174c4beb97e99b8aabcf9dbc1ce3b28d3f4fb849a44298655202bf3eff8dcd8ed1cf4ed8baa9e7bc6bc6648ccfb5905da6d7e9b92d745c24422e2d86ada8ebb2b5bf51d8d3598c4b227f7ee906e045fcb202153aa88891e18886fcbe20c1f3284bcf57ed7a862c8823d2275f237d68430deef6fe639a33e105e9612b6b9adc3f9d59fa3f1455b1ec67148d8ffb018cc64188423b9c70732c38553a8d11a87bac2c91d3ad5e1aeacea728789cfd47c8a9f8888b12f10e0133cd0f186019bf18611baabe86e393c72774b2cff2990d9bdac66033c695651b0e4745dae3c7c0361ba5001c003bdceb09038c213072ba25d2a6ad258242f89ef7869a20a179d658efabca05ed124d668044156ad5184b7abbb86739979154df40ce2ead0f0f69ffad3484d0f1b72e793734f8ad55d7a0e922ba3b9d21b84e470c0bcbd4a5bba5fa7517788aa475e834133846a086b685f1aacf3795b642777e49456296bf200d6bf5daf3dc039c245234ce505eef3eaf8e66d92a79f0b1b46ea1d39ffc66dad071525bb5b3c5288bd10566a7c18754e47f24fcc61c407c989080cae9544e1d8f91a6cf21a4a9e2fd057cfc3e2d2d8f856fc8662cb4a3530dd4dd373835324a705800cb8dda6e3fc41e9cd9e3643b6314fae98b3e714affe0aff4dd0398731e9f820789fb80e54ce1678de9412c772a1a2299217e576945d116dae37cd1d63552c858830f22e76a3fbc77d9394ceb237c256123439d6ff8b7e086ae3bd2275ffd6e9d3cddcc1dce0a385d0a49a5319dace0fe5b7a0950c1c0e4e2be35376b6bf48c4f1982b3594c999a5db0ef91131c13f1387bb3ac6178035ea9b8c100754b2d69c0d26d5f0a5379126e0425d565eac08ba1de88e1370a5510d5dd38fc37065e012273e5fd0b926c86e18e6b14af474b4d953d43b3704d437a6dee615da75c51f60e693fb8fbdef2fe40d6c147100392ba8dca8637dbff7e55b3b6e2a7cbc8184f35ded11a0a54a7582dfb514886665879fa721596a69071cecd4eecfece310935d30a6da00a44a2f6ac4475638449dcb205be0a3df233b476f3169d40334a36e0499eeab4ad36334ff1ec3c651dce5e197d0f50b973d1b06fb9e2f5162ac1453ec4c373cb276d03ce877b59bf4ac8d6c0f89143e3e947abd1d0644ecc910d8bb6fd888e1db33898113d24a3bfaf41546952e2074dec0bc8e216ac44c2d3755c48c79e2439e89e67df32fd494b5f45ee1f55cad4a896b2997dd7300efac1c0

- No. of header fields: 4

- Names of fields:

- Source Port (udp.srcport)

- Destination (udp.dstport)

- Length (udp.length)

- Checksum (udp.checksum)

- Each UDP header field is 2 bytes (16 bits) long. With 4 fields—Source Port, Destination Port, Length, and Checksum—the total UDP header size is 8 bytes.

- The Length field indicates the total size of the UDP segment (header + payload). In my captured packet, the Length is 1069 bytes, and the UDP header is 8 bytes, so the UDP payload is 1069 − 8 = 1061 bytes, which matches the payload shown in Wireshark.

- The maximum size of a UDP segment is 65,535 bytes. Subtracting the 8-byte header, the maximum UDP payload is 65,527 bytes.

- The Source Port field is 16 bits, so the largest possible source port number is 65,535.

- The protocol number for UDP is 17 in decimal notation.

- In my trace file, the first UDP packet is Packet Number 197, and the second UDP packet is Packet Number 270. The source port (60282) of the first packet is the destination port of the second packet, and the destination port (443) of the first packet is the source port of the second packet, which shows the reply relationship between the two packets.